Test Cases & Results for ROS2 as middleware

- Module Definition

  - Data node

    - D1: Image node

      - PS: Publish the 1920*1080pixels / 60fps image stream a

      - Service: Start/Stop

    - D2: Image Process node

      - PS: Subscribe image stream a

      - PS: Publish Status String b / Result String c

      - Service: Start/Stop/Image Process Rate(control the resources of processing)

    - D3: Result Convert node

      - PS: Subscribe Result String c

      - OUTPUT: UDP package d

  - Log node

    - L1: Passive Log node

      - PS: Subscribe Status String b

      - PS: Publish log e

    - L2: Active Log node

      - Action: Push log f alternatively

    - L3: Log Convert node

      - PS: Subscribe log e / f

      - OUTPUT: UDP package g

  - Script

    - S1: A launch script for system

      - Launch D1->D2->D3

      - Launch L1, L2, L3

    - S2: A stop script for system

Test cases for developers

Stability and Robustness

- Case 1: Single Long Time Run

  - Image Process Rate to be set to 1.0

  - Launching S1, to run the system for x hours

  - Checking stuck/crashing status

- Standard 1: x should be larger than 24 without system crashing/stuck

- Test Steps:

Run the following nodes in order.

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg D2cpp

③ros2 run test_pkg D3cpp

④ros2 run test_pkg L1

⑥ros2 run test_pkg L2

⑦ros2 run test_pkg L3

⑧ros2 run test_pkg talker / ros2 run test_pkg listener

Figure 1. rqt_graph

-   Results:

Time of running: 93725 seconds;  Crashing Error: 0; Stuck Error: 0

①The node network is still communicating normally, and there is no crashed node and the talker and listener still work well;

②At 93890s, through instruction `ros2 topic echo “/D2cpp_pub” (“/pub_e /chatter …”)`, The data shows correctly and normally.

Through the rviz2, we can see that the topic of “/image_source_raw” and the video stream still runs well.

- Case 2: Loop Long Time Run

  - Image Process Rate to be set to 1.0

  - Launching S1, to run the system for x hours

  - Launching S2, then launching S1, looping for y times

  - Checking stuck/crashing status

- Standard 2: y should be larger than 1000 without system crashing/stuck

- Test Steps:

① Use a shell script to automatically launch S1 (to start all nodes) and S2 (to stop all nodes). Command: `./test_s1_s2.sh

- Results:

Cycles: 1200 times;  Crashing Error: 0; Stuck Error: 0

①The node network is still communicating normally, and there is no crashed node. The talker and listener still works well.

- Case 3: Broken Run

  - Image Process Rate to be set to 1.0

  - Launching S1, to run the system for x1 hours

  - Killing Middleware service for y times

    - kill L1/L2, check output of string c

    - kill D3, check L1/L2 log

  - Checking stuck/crashing status and measuring this period as x2 hours

- Standard 3: x2 should be larger than 0.25 without system crashing/stuck

- Test Steps:

Run the following nodes in order:

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg D2cpp

③ros2 run test_pkg D3cpp (will be killed later...)

④ros2 run test_pkg L1

⑥ros2 run test_pkg L2 (will be killed later...)

⑦ros2 run test_pkg L3

⑧ros2 run test_pkg talker / ros2 run test_pkg listener

Figure 2. killing L2 node, rqt_graph

Figure 3. killing L2&D3 node, rqt_graph

- Results:

Time of running: 1 hours; Crashing Error: 0; Stuck Error: 0

①Kill the node  L2, and wait for about 0.3 hours, then restart the L2. Re-establishment of communication between nodes happened;

②Kill the node  D3, and wait for about 0.3 hours, then restart the D3. Re-establishment of communication between nodes happened.

System Resource

- Case 4: Low Resource Run

  - Image Process Rate to be set to k * 1.0, making the CPU occupancy rate r1 / GPU occupancy rate r2

  - Launching S1, to run the system for x hours

  - Checking stuck/crashing status

- Standard 4: when r1/r2 is larger than 80%, x should be larger than 1 without system crashing/stuck

- Test Steps:

run the following nodes in order:

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg client_rateChange

(which makes the image process rate change according to k*1.0)

③ros2 run test_pkg client_time

(which makes the CPU occupancy rate change dynamically from 1-8 CPU utilization processes every hour)

- Note:

Use command `cat /proc/cpuinfo | grep 'processor' |wc -l` to check the status of CPUs

Figure 4. the number of cpu to occupy

Figure 5.Low Resource Run rqt_graph

Figure 6.cpu processes usage (6/8)

- Results:

Time of running: 18825 seconds; Error: 0; Warning: 0

①The node network is still communicating normally, and there is no crashed node. The talker and listener still works well;

②At 18890s, Through the rviz2, we saw that the topic of“/image_source_raw”and the video stream was still running well;

③Through command `ros2 run test_pkg client_time` that published clients to control CPU rate, in case of high CPU usage (5-8 CPU utilization processes), all of nodes still work well.

- Case 5: High Concurrency Run

  - Duplicating D1->D2->D3 for y1 times, marking the CPU occupancy rate r1 / GPU occupancy rate r2

  - Launching S1, to run the system

  - Measuring the publisher and subscriber I/O operations per second as y2

  - Checking stuck/crashing status

- Standard 5: when r1/r2 is larger than 80%, y1 should be larger than 8, y2 should be larger than () without system crashing/stuck

- Test Steps:

Run the following nodes in order.

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg D2cpp

③ros2 run test_pkg D3cpp

④ros2 run test_pkg L1

⑥ros2 run test_pkg L2

⑦ros2 run test_pkg L3

⑧ros2 run test_pkg talker　ros2 run test_pkg listener

⑨ros2 run test_pkg client_time

(which makes the CPU occupancy rate change dynamically from 1-8 CPU utilization processes every half an hour)

Figure 7.ros2 node list

Figure 8. High Concurrency rqt_graph

Figure 9.cpu processes usage (５/8)

Figure 10.cpu processes usage (７/8)

-   Results:

Time of running: 93725 seconds; Error: 0; Warnings: 1

①Look at Figure 9 and 10, the CPU processes usage changes from 5/8 to 7/8 which made the CPU utilization allocated to the node of image_source  was reduced from 75.7% to 56.7%, and to the node of D2cpp was reduced from 2.7% to 1.0%.

②The node network is still communicating normally, and there is no crashed node and the talker and listener still worked well;

③At 93890s, through command `ros2 topic echo “/D2cpp_pub” (“/pub_e /chatter …”)`, the data showed correctly and normally.

Through the rviz2, we saw that the topic of “/image_source_raw”and the video  stream was still running well;

④ At 93932s, running command `ros2 run test_pkg client 0 1(2,3,4….) ` to publish clients that changed the number of CPU rate, it can still deliver messages normally and work well.

⑤At about > 93725 seconds, there was a warning showing. The warning was from “ros2 run test_pkg client_time”, but it did not affect the normal communication of the network. Restarting this node can solve this.

Figure 11.The node “ client_time” warning

Test cases for user experience

Eligibility

- Case 6: Low Latency Run

  - Image Process Rate to be set to 1.0

  - Launching S1, to run the system for x hours

  - Measuring the system latency from D1 to D3 as y ms

- Standard 6: y should be less than 30

-  Test Steps:

Run the following nodes in order.

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg D2cpp

③ros2 run test_pkg D3cpp

-  Result:

Time of running: 10 min; Error: 0; Warnings:0

Figure 12.The node “Image_source_video” Latency test

①From Figure 12, the average latency of the node “image_source_video” was about 30 ms.

Usability

- Case 7: Interruption Run

  - Image Process Rate to be set to 1.0

  - Launching S1, to run the system for x hours

  - Call Service of S1/S2 for y times within one reaction period

  - Checking stuck/crashing status

- Standard 7: when y is larger than 2, system should not be crashing/stuck

- Test Steps:

Run the following nodes in order.

①ros2 run py_pubsub image_source_video (D1py)

②ros2 run test_pkg D2cpp

③ros2 run test_pkg D3cpp

④ros2 run test_pkg L1

⑥ros2 run test_pkg L2（Push log f alternatively）

⑦ros2 run test_pkg L3  (Accept)

⑧ros2 run test_pkg talker / ros2 run test_pkg listener

⑨ros2 run test_pkg client_time

(which makes the CPU occupancy rate change dynamically from 1-8 CPU utilization processes every second)

-  Result:

Time of running: 12528 seconds; Error:0; Warning:0

①The node network was still communicating normally, and there was no crashed node and the talker and listener still worked well.

- Case 8: Data frame loss test

| Msg : “Hello LinSC666! Test wheather LosingData...msg->data”+int[num] The number of Msg: 1,000,000 CPU processes usage: 6/8 && 1/8 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Sending Frequency |  |  |  |
|  | 100Hz | 500Hz | 1KHz | 10KHz |
| Number of dropped frames | 999982/1000000 (miss 18) | 999936/1000000 (miss 64) | 999896/1000000 (miss 104) | 571209/1000000 (miss 428791) |
|  |  |  |  | 998973/1000000 (Low CPU processes usage: 1/8,miss 1027) |

Figure 13.Number of dropped frames under 100Hz,6/8 cpu usage

Figure 14.Number of dropped frames under 500Hz,6/8 cpu usage

Figure 15.Number of dropped frames under 1KHz,6/8 cpu usage

Figure 16.Number of dropped frames under 10KHz,6/8 cpu usage（ serious data lost)

Figure 17.Number of dropped frames under 10KHz,1/8 cpu usage

-  Result:

①under CPU processes usage= 6/8，when Sending Frequency <=1KHz, it will lose little some data frame.

②under CPU processes usage= 6/8,when Sending Frequency >=10KHz, it lost a lot of data frames,and it occurred serious data lost.But it showed good  under CPU processes usage= 1/8.

| Topic : “image_source/image_raw” The number of Frame_id: 1,000,000 Picture resolution : 980 * 720 CPU processes usage: 6/8 && 1/8 |  |  |  |
| --- | --- | --- | --- |
|  | Sending Frequency |  |  |
|  | 100Hz | 500Hz | 1KHz |
| Number of dropped frames | 999976/1000000 (high CPU processes usage: 6/8,miss 24) | 997248/1000000 (high CPU processes usage: 6/8,miss 2752) | 992400/1000000 (high CPU processes usage: 6/8,miss 7600) |
|  |  |  | 997968/1000000 (Low CPU processes usage: 1/8,miss 2032) |

Figure 18.The Received Image show

Figure 19.Number of dropped image_frames under 100Hz,6/8 cpu usage

Figure 20.Number of dropped image_frames under 500Hz,6/8 cpu usage

Figure 21.Number of dropped image_frames under 1KHz,6/8 cpu usage

Figure 22.Number of dropped image_frames under 1KHz,1/8 cpu usage

-  Result:

①Each image resolution was 980*720, under CPU processes usage= 6/8，when Sending Frequency <=100Hz, it will lose little some image frame.

②under CPU processes usage= 6/8,when Sending Frequency >=500Hz, it lost a lot of image frames,and it occurred serious data lost.But it showed good  under CPU processes usage= 1/8.